Gear Parameters

- Module (m): 2 mm

- Pressure Angle: 20°

- Number of Planet Gears: 3

Teeth Count

| Gear | Teeth |
| --- | --- |
| Sun Gear | 12 |
| Planet Gear | 24 |
| Ring Gear | 60 |

This gives a 4:1 reduction ratio when the ring gear is fixed.

Sun Gear

Dimensions

| Parameter | Value |
| --- | --- |
| Teeth | 12 |
| Module | 2 mm |
| Pitch Diameter | 24 mm |
| Outside Diameter | 44 mm |
| Bore Diameter | 10 mm |
| Face Width | 10 mm |

Pitch diameter:

d=mz

For Sun Gear:

- d = 2 × 12 = 24 mm

Planet Gear (3x)

| Parameter | Value |
| --- | --- |
| Teeth | 24 |
| Module | 1 mm |
| Pitch Diameter | 40 mm |
| Outside Diameter | 44 mm |
| Bore Diameter | 8 mm |
| Face Width | 15 mm |

Ring Gear

| Parameter | Value |
| --- | --- |
| Teeth | 60 |
| Module | 2 mm |
| Pitch Diameter | 120 mm |
| Root Diameter | 116 mm |
| Outer Diameter | 140 mm |
| Face Width | 15 mm |

Make the ring thickness:

- 10 mm radial thickness

This makes it robust and easy to model.

Planet Carrier

Main Plate

| Parameter | Value |
| --- | --- |
| Diameter | 90 mm |
| Thickness | 3.5 mm |

Planet Pin Locations

Planet center radius:

30 mm

Place three holes:

- 120°

- 240°

- 360°

Hole diameter:

5 mm

Planet Pins

| Parameter | Value |
| --- | --- |
| Diameter | 8 mm |
| Length | 25 mm |

Input Shaft

| Parameter | Value |
| --- | --- |
| Diameter | 10 mm |
| Length | 80 mm |

Output Shaft

| Parameter | Value |
| --- | --- |
| Diameter | 12 mm |
| Length | 80 mm |

Housing

Outer Cylinder

| Parameter | Value |
| --- | --- |
| Outer Diameter | 150 mm |
| Length | 30 mm |

Mounting Holes

Use:

- 4 holes

- M6

- Equally spaced

Bearings (Optional)

You can either:

- Model simplified bearings, or

- Ignore them for Version 1

For a portfolio project, simplified bearings are enough.

Final Assembly Size

Approximate dimensions:

Diameter ≈ 150 mm
Length ≈ 30 mm

This is large enough to show detail and small enough to manage in SolidWorks.

Recommended Part List

Sun_Gear.SLDPRT
Planet_Gear.SLDPRT
Ring_Gear.SLDPRT
Carrier.SLDPRT
Planet_Pin.SLDPRT
Input_Shaft.SLDPRT
Output_Shaft.SLDPRT
Housing.SLDPRT

That's 8 parts, which is a very good size for a student portfolio project. It will let you demonstrate:

- Part modeling

- Assemblies

- Circular patterns

- Motion studies

- Drawings

- BOM generation

without becoming overwhelming.